EV97 CHECK LIST

# PRE-START CHECKS

CONFIRM OK TO START WHERE PARKED

S = SECURITY. HATCHES, HARNESS, BRAKES ON.

T = THROTTLE CLOSED, FUEL ON.

A = ALL CLEAR AROUND AIRCRAFT. EVENTUALITIES

M = MASTER SWITCH ON.  IGNITION ON, AVIONICS OFF.

P = CALL ‘CLEAR PROP’ THEN COVERING THROTTLE AND               WITH CHOKE IF REQUIRED, START ENGINE, SET 2000RPM

C = CHARGING SWITCH ON. CHECK VOLTMETER SHOWING A CHARGE .  CHECK OIL PRESSURE IN GREEN, AVIONICS ON

#### PRE-TAKE OFF CHECKS

C = CONTROLS, FULL AND FREE MOVEMENT.

CHECK CORRECT SENSE.

H = HATCH AND HARNESS SECURE

I = INSTRUMENTS CHECKED AND SET, LIGHT ON,

TRANSPONDER SET,  SKYECHO ON, GARMIN GPS ON

F = FUEL ON. CHOKE OFF, CONTENTS SUFFICIENT.

A = ALL CLEAR AROUND AIRCRAFT

P = POWER CHECK (3500 RPM) AND IGNITION CHECK. IDLE CHECK

F = FLAPS SET FOR TAKE OFF.  (One notch)

T = TRIM SET FOR TAKE OFF

W = WIND. ASSESS SPEED AND DIRECTION (Within limits)

A = (AT THE HOLD) CHECK ALL CLEAR FOR DEPARTURE

ON ROUTE CHECKS

L = LOCATION

I = INSTRUMENTS CHECKED

F = FUEL SUFFICIENT

E = ELAPSED TIME

#### AIRFIELD APPROACH CHECKS

F = FUEL SUFFICENT

A = ALTIMETER SET

R = RUNWAY IN USE

S = SQUAWK CODE SET

PRE-LANDING CHECKS

F = FUEL SUFFICENT FOR APPROACH AND GO AROUND

A = ALTIMETER SET QFE AND CONFIRM CIRCUIT HEIGHT

W = WIND ASSESS SPEED AND DIRECTION

T = TRAFFIC IN CIRCUIT

H = HATCH AND HARNESS SECURE

#### PRE-STALL CHECKS

# H =  HEIGHT SUFFICENT TO

# RECOVER BY 1000 AGL

A = AIRFRAME CLEAN CONFIGURATION

S = SECURITY OF HARNESS. NO LOOSE ITEMS.

E = ENGINE TEMPRETURES OK

L = LOCATION

A = CLEAR OF ACTIVE AIRFIELD, AIRSPACE

B = CLEAR OF BUILT-UP AREAS

C = CLEAR OF CLOUD

L = LOOKOUT WITH CLEARING TURN

EMERGENCY CHECKS

T =THROTTLE CLOSED

I = IGNITION OFF

F = FUEL OFF

S = HARNESS SECURE

# AFTER LANDING CHECKS

F = FLAPS RETRACTED.

S = LANDING LIGHT OFF.

------------------------------------------------------------------------------

SHUT DOWN CHECKS

S = SWITCH OFF AVIONICS

M = IGNITION CHECK AT 2000RPM

M = IGNITION OFF

M = MASTER SWITCH OFF, CHARGING SWITCH OFF.

----------------------------------------------------------------------------------------------Updated SEPTEMBER 2022